This document describes the info registers of the DCF for the Genesis grab module.

INFO_XSIZE:

This info register contains the width of the active grab window in pixel.

INFO_YSIZE:

This info register contains the height of the active grab window in pixel.

INFO_MODE:

Type of the scan mode.

CAM_INFO_NON_INTERLACED:

The camera is non-interlaced (mandatory for line scan).

Value: 0

CAM_INFO_INTERLACED:

The camera is interlaced.

Value: 1

INFO_TYPE:

Type of the camera.

CAM_INFO_LINE_SCAN:

The camera is line scan.

Value: 1

CAM_INFO_FRAME_SCAN:

The camera is frame scan. The definition of the frame scan in this context is a camera that send a VSYNC at the beginning of a frame but not at the end. It is viewed as a non-interlaced camera with a asynchronous reset. We must count the number of lines to know when the frame is completely grabbed.

Value: 2

CAM_INFO_AREA_SCAN:

The camera is area scan. This means that a VSYNC is present at the beginning and at the end of each frame.

Value: 3

CAM_INFO_OTHER_SCAN:

A special type a scanning. Not determined yet.

Value: 4

INFO_INPUT:

Type of the input data.

CAM_INFO_DATA_ANALOG:

The data is from the analog port.

Value: 0

CAM_INFO_DATA_TTL:

The data comes from the camera in digital TTL format.

Value: 4

CAM_INFO_DATA_RS422:

The data comes from the camera in digital RS-422 format.

Value: 5

INFO_PIXCLK:

The pixel clock frequency given in hertz.

INFO_SPIXCLK:

The pixel clock frequency, in hertz, at which the PLL must be prelocked.

INFO_CONTPCLK:

Indicates if the pixel clock is continuous or not.

CAM_INFO_NON_CONTINUOUS_PCLK:

The pixel clock is not continuous.

Value: 0

CAM_INFO_CONTINUOUS_PCLK:

The pixel clock is continuous.

Value: 1

INFO_USRCLK0:

User clock 0 frequency in hertz.

INFO_USRCLK1:

User clock 1 frequency in hertz.

INFO_USRCLK2:

User clock 2 frequency in hertz.

INFO_USRCLKSEL:

Select the user clock.

CAM_INFO_USERCLOCK_0:

Select the user clock 0.

Value: 1

CAM_INFO_USERCLOCK_1:

Select the user clock 1.

Value: 2

CAM_INFO_USERCLOCK_2:

Select the user clock 2.

Value: 3

INFO_GRBSYSCLK:

The noise gating system clock frequency in hertz.

INFO_SAMPLEMODE:

Select the sampling mode.

CAM_INFO_4_CHANNEL_8_BITS:

Sample with four 8 bits channels.

Value: 0

CAM_INFO_2_CHANNEL_8_BITS:

Sample with two 8 bits channels.

Value: 1

CAM_INFO_1_CHANNEL_8_BITS:

Sample with a single 8 bits channel.

Value: 2

CAM_INFO_1_CHANNEL_10_BITS:

Sample with a single 10 bits channel.

Value: 3

INFO_CHANNEL:

Select the logical channel.

CAM_INFO_NO_CHANNEL:

Use no channels.

Value: 0

CAM_INFO_CHANNEL_0:

Use the channel 0.

Value: 0x10000

CAM_INFO_CHANNEL_1:

Use the channel 1.

Value: 0x20000

CAM_INFO_CHANNEL_2:

Use the channel 2.

Value: 0x40000

CAM_INFO_CHANNEL_3:

Use the channel 3.

Value: 0x80000

CAM_INFO_CHANNEL_01:

Use the channel 0 and 1.

Value: 0x30000

CAM_INFO_CHANNEL_02:

Use the channel 0 and 2.

Value: 0x50000

CAM_INFO_CHANNEL_03:

Use the channel 0 and 3.

Value: 0x90000

CAM_INFO_CHANNEL_12:

Use the channel 1 and 2.

Value: 0x60000

CAM_INFO_CHANNEL_13:

Use the channel 1 and 3.

Value: 0xa0000

CAM_INFO_CHANNEL_23:

Use the channel 2 and 3.

Value: 0xc0000

CAM_INFO_CHANNEL_012:

Use the channel 0,1 and 2.

Value: 0x70000

CAM_INFO_CHANNEL_013:

Use the channel 0,1 and 3.

Value: 0xb0000

CAM_INFO_CHANNEL_023:

Use the channel 0,2 and 3.

Value: 0xd0000

CAM_INFO_CHANNEL_123:

Use the channel 1,2 and 3.

Value: 0xe0000

CAM_INFO_CHANNEL_0123:

Use the channel 0,1,2 and 3.

Value: 0xf0000

INFO_BITSPERLOGCH:

The width, in bit, of a logical channel. All the selected channels will have the same width.

INFO_TIMEMULTICH:

Total clock cycles before all time-multiplexed channels of a camera are received on the same byte lane.

INFO_TIMEMULTIPIX:

Total clock cycles before all time-multiplexed bytes of a pixel are received on the same byte lane.

INFO_XTAPSPERCH:

Non-consecutive pixels per clock cycle, per channel, in the x direction.

Values:

- 1: There is no non-consecutive pixels. This is the default value.

- 2: The grab is done in two columns. One beginning at x=0 and the other beginning at x=HTOTAL/2.

- 4: The grab is done in four columns. HTOTAL is divided in four quarters.

0						512					1024

TAP1						TAP2

INFO_YTAPSPERCH:

Non-consecutive pixels per clock cycle, per channel, in the y direction.

Values:

- 1: There is no non-consecutive pixels. This is the default value.

- 2: The grab is done in two rows. One beginning at the first line and the other beginning at y=VTOTAL/2.

- 4: The grab is done in four rows.

TAP1     0

TAP2    512

INFO_XTAPSPERCHADJ:

Adjacent pixels per clock cycle, per channel, in the x direction.

Values:

- 1: There is no adjacent pixel. This is the default value.

- 2: There is 2 adjacent pixels. At each clock cycle, we grab the even and the odd pixel of a line ie 0 and 1, 2 and 3…

- 4: There is 4 adjacent pixels. At each clock cycle, we grab four consecutive pixel of a line ie 0,1,2 and 3, 4,5,6 and 7…

0           1

TAP1  TAP2

INFO_YTAPSPERCHADJ:

Adjacent pixels per clock cycle, per channel, in the y direction.

Values:

- 1: There is no adjacent pixel in the y direction. This is the default value.

- 2: There is 2 adjacent pixels. We grab the even and the odd row at the same time.

- 4: There is 4 adjacents pixels. We grab four consecutive lines at the same time.

TAP1     0

TAP2     1

INFO_PIXPERCLOCK:

The number of consecutive pixels per clock cycle, per channel.

INFO_PACKEDPIXELS:

Used to identify packed RGB format for the VIA.

CAM_INFO_NORMAL_RGB:

Only received one pixel per clock tick.

Value: 0

CAM_INFO_PACK_RGB:

Used when the RGB input is packed.

Value: 1

INFO_SYNCGRABCHAN:

Used to determine if the sync channel is the same as the grab channel. Used only in analog.

CAM_INFO_SYNC_INDEPENDANT:

If we change the grab channel, the sync channel remain the same.

Value: 0

CAM_INFO_SYNC_FOLLOW:

If we change the grab channel, the sync channel will change.

Value: 1

INFO_LUTBUFID:

The LUT buffer ID. Must be set to 0 for transparent LUT.

à Must always be set to 0.

INFO_LUTPROG:

Flag to determine if the luts should be programmed of not.

CAM_INFO_DEFAULT_LUT:

Don’t program the LUT.

Value: 0

à Must always be set to 0.

CAM_INFO_PROGRAM_LUT:

Program the LUT.

Value: 1

INFO_T1DELAY:

The timer 1 delay in timer clock source ticks.

INFO_T2DELAY:

The timer 2 delay in timer clock source ticks.

INFO_TRIGSRC:

Trigger source of the VIA capture signal.

CAM_INFO_TRIGGER_SOFTWARE:

Use the software trigger.

Value: 0

CAM_INFO_TRIGGER_VIDEO_CONNECTOR:

Use the input of the video connector(analog) as the trigger.

Value: 0x100001

CAM_INFO_TRIGGER_DIGITAL_CONNECTOR:

Use the input of the digital connector as the trigger.

Value: 0x200001

CAM_INFO_TRIGGER_TIMER_1:

Use the output of timer 1 as the trigger.

Value: 0x10004

CAM_INFO_TRIGGER_TIMER_2:

Use the ouput of timer 2 as the trigger.

Value: 0x20004

CAM_INFO_NO_TRIGGER:

Don’t use a trigger.

Value: 0x20000000

INFO_T1TRGSRC:

The trigger source for the timer 1.

CAM_INFO_TRIGGER_SOFTWARE:

Use the software trigger.

Value: 0

CAM_INFO_TRIGGER_VIDEO_CONNECTOR:

Use the input of the video connector(analog) as the trigger of the timer.

Value: 0x100001

CAM_INFO_TRIGGER_DIGITAL_CONNECTOR:

Use the input of the digital connector as the trigger of the timer.

Value: 0x200001

CAM_INFO_TRIGGER_VSYNC:

Use the VSYNC as the trigger of the timer.

Value: 2

CAM_INFO_TRIGGER_HSYNC:

Use the HSYNC as the trigger of the timer.

Value: 3

CAM_INFO_CONTINUOUS:

Use the end of the count of the timer as the trigger.

Value: 0xffffffff

CAM_INFO_TIMER_2:

Use the ouput of timer 2 as the trigger.

Value: 0x20000

CAM_INFO_NO_TRIGGER:

Don’t use a trigger.

Value: 0x20000000

INFO_T2TRGSRC:

The trigger source for the timer 2.

CAM_INFO_TRIGGER_SOFTWARE:

Use the software trigger.

Value: 0

CAM_INFO_TRIGGER_VIDEO_CONNECTOR:

Use the input of the video connector(analog) as the trigger of the timer.

Value: 0x100001

CAM_INFO_TRIGGER_DIGITAL_CONNECTOR:

Use the input of the digital connector as the trigger of the timer.

Value: 0x200001

CAM_INFO_TRIGGER_VSYNC:

Use the VSYNC as the trigger of the timer.

Value: 2

CAM_INFO_TRIGGER_HSYNC:

Use the HSYNC as the trigger of the timer.

Value: 3

CAM_INFO_CONTINUOUS:

Use the end of the count of the timer as the trigger.

Value: 0xffffffff

CAM_INFO_TIMER_1:

Use the ouput of timer 1 as the trigger.

Value: 0x10000

CAM_INFO_NO_TRIGGER:

Don’t use a trigger.

Value: 0x20000000

INFO_T1CLKSRC:

Indicates which clock we may select for the timer 1.

Combination of any of the following (OR).

CAM_INFO_CLK_DITRIG:

The clock source is the digital connector.

Value: 1

CAM_INFO_CLK_VITRIG:

The clock source is the video connector.

Value: 2

CAM_INFO_CLK_OHSYNC:

The clock source is the HSYNC signal.

Value: 4

CAM_INFO_CLK_OVSYNC:

The clock source is the VSYNC signal.

Value: 8

CAM_INFO_CLK_EXP2:

The clock source is the exposure 2 output.

Value: 0x10

CAM_INFO_CLK_USRCLK:

The clock source is the user clock.

Value: 0x20

CAM_INFO_CLK_PIXCLK:

The clock source is the pixel clock.

Value: 0x40

CAM_INFO_CLK_XTALCLK:

The clock source is the crystal clock(28.636MHz).

Value: 0x80

CAM_INFO_CLK_ALL:

Enable all the clock source. Use this value if there is no trigger for timer 1.

Value: 0xff

INFO_T2CLKSRC:

Indicates which clock we may select for the timer 2.

Combination of any of the following (OR).

CAM_INFO_CLK_DITRIG:

The clock source is the digital connector.

Value: 1

CAM_INFO_CLK_VITRIG:

The clock source is the video connector.

Value: 2

CAM_INFO_CLK_OHSYNC:

The clock source is the HSYNC signal.

Value: 4

CAM_INFO_CLK_OVSYNC:

The clock source is the VSYNC signal.

Value: 8

CAM_INFO_CLK_EXP1:

The clock source is the exposure 1 output.

Value: 0x10

CAM_INFO_CLK_USRCLK:

The clock source is the user clock.

Value: 0x20

CAM_INFO_CLK_PIXCLK:

The clock source is the pixel clock.

Value: 0x40

CAM_INFO_CLK_XTALCLK:

The clock source is the crystal clock(28.636MHz).

Value: 0x80

CAM_INFO_CLK_ALL:

Enable all the clock source. Use this value if there is no trigger for timer 1.

Value: 0xff

INFO_T1EXPMOD:

Flag to permit the modification of the timer 1 timings.

CAM_INFO_LOCK_TIMER1:

Cannot modify the timer 1 timings.

Value: 0

CAM_INFO_MODIFY_TIMER1:

Can modify the timer 1 timings.

Value: 1

INFO_T2EXPMOD:

Flag to permit the modification of the timer 2 timings.

CAM_INFO_LOCK_TIMER2:

Cannot modify the timer 2 timings.

Value: 0

CAM_INFO_MODIFY_TIMER2:

Can modify the timer 2 timings.

Value: 1

INFO_XYSIZEMOD:

Flag to permit the modification or not of the x and/or y size.

CAM_INFO_LOCK_XYSIZE:

Cannot modify the x and y size.

Value: 0

CAM_INFO_MODIFY_XSIZE:

Can modify only the x size.

Value: 1

CAM_INFO_MODIFY_YSIZE:

Can modify only the y size.

Value: 2

CAM_INFO_MODIFY_XYSIZE:

Can modify both x and y size.

Value: 3

INFO_CLAMP:

Flag indicating if the clamping signal should be generated.

CAM_INFO_GEN_CLAMP:

Generate the clamping signal. This is the default.

Value: 0

CAM_INFO_NO_GEN_CLAMP:

Don’t generate the clamping signal.

Value: 1

CAM_INFO_AUTO_CLAMP:

The generation of the siganl is automatic.

Value: 2

INFO_MISC:

This register will be defined later.

INFO_LINEDELAY:

Number of lines of offset between the three channels of a color line scan camera. The value should be 0 for all the other cases.